T4i _ SCR FAULT CODE LISTING

test bed/test  MR-PLD/55/B_MR2_SW23

description file: C:\Technical Service and Parts\Mecedes\Mini-Diag\5.02.61\MD2_5.02.61_DIAGNOSTIC_2011-05-13\MR-PLD.ECU\5254fbbb.DAT

parameter file: C:\Technical Service and Parts\Mecedes\Mini-Diag\5.02.61\MD2_5.02.61_DIAGNOSTIC_2011-05-13\test bed.CUS\403751e3.PAR

text file: C:\Technical Service and Parts\Mecedes\Mini-Diag\5.02.61\MD2_5.02.61_DIAGNOSTIC_2011-05-13\MR-PLD.ECU\5254fbbb.TXl

code memory fault path 01 CAN connection (low speed)

code	description

0100	CAN-high defect

0101	CAN-low defect

0102	CAN data not plausible

0104	no connection to CAN

0149	CAN parameter setting fault

code memory fault path 02 CAN connection (high speed)

code	description

0204	no connection to CAN

code memory fault path 03 crankshaft sensor

code	description

0308	shortened to ground

0309	interruption

0310	signal level to low

0311	not plausible with camshaft sensor

0312	crankshaft signal time out

0313	sensor connector mixed up

code memory fault path 04 camshaft sensor

code	description

0408	shortened to ground

0409	interruption

0412	camshaft sensor signal time out

0413	sensor connector mixed up

code memory fault path 05 engine

code	description

0530	engine speed above limit

code memory fault path 06 fan speed

code	description

0612	signal timeout

0631	engine speed to low

code memory fault path 07 SCR catalyst

code	description

0722	temperature to high (after catalyst)

0782	component (catalyst) not available

0788	temperature before catalyst to high - threshold 1 out of range, to high

0789	temperature before catalyst to high - threshold 2 out of range, to high

0797	temperature before catalyst to high (torque limiting)

code memory fault path 08 SCR catalyst - temperature sensor 1

code	description

0815	out of range, to high

0816	out of range, to low

code memory fault path 09 SCR catalyst - temperature sensor 2

code	description

0915	out of range, to high

0916	out of range, to low

code memory fault path 10 oil temperature sensor

code	description

1015	out of range, to high

1016	out of range, to low

code memory fault path 11 fuel temperature sensor

code	description

1115	out of range, to high

1116	out of range, to low

code memory fault path 12 boost temperature sensor

code	description

1215	out of range, to high

1216	out of range, to low

code memory fault path 13 ambient pressure sensor

code	description

1315	out of range, to high

1316	out of range, to low

code memory fault path 14 boost pressure sensor

code	description

1415	out of range, to high

1416	out of range, to low

1417	value not plausible

code memory fault path 15 coolant temp. sensor

code	description

1515	out of range, to high

1516	out of range, to low

code memory fault path 16 oil pressure sensor

code	description

1615	out of range, to high

1616	out of range, to low

1617	signal not plausible

code memory fault path 17 combination input (FPS,P3,P-DK)

code	description

1715	out of range, to high

1716	out of range, to low

1717	signal not plausible

code memory fault path 18 booster path

code	description

1818	boost circuit fault

1820	boost air pressure, to high

1822	boost air temperature, to high

1873	synchronizer control stop

1874	boost control: set value deviation, to high

1875	boost control: boost deviation, to high

1876	boost control: brake power, to low

1878	bypass valve - mechanical

code memory fault path 19 fuel circuit

code	description

1917	value not plausible

code memory fault path 20 oil circuit

code	description

2021	oil pressure, to low

2026	engine-oil level, to high or to low

code memory fault path 21 engine coolant circuit

code	description

2122	temperature, to high

code memory fault path 22 term. 15 detection

code	description

2219	inconsistence MR <-> FR

code memory fault path 23 term. 50 detection

code	description

2319	inconsistence MR <-> FR

code memory fault path 24 scavenging gradient (P2S - P3)

2415	out of range, to high

2416	out of range, to low

code memory fault path 25 oil level sensor

code	description

2509	interruption

2515	out of range, to high

2516	out of range, to low

code memory fault path 26 booster speed sensor 1

code	description

2612	no speed (time out)

2615	out of range, to high

2616	out of range, to low

code memory fault path 27 booster speed sensor 2

code	description

2712	no speed (time out)

code memory fault path 28 NOx sensor

code	description

2804	no communication

2816	out of range, to low

2817	out of range, to high

code memory fault path 29 NOx emission

code	description

2971	threshold 1 exceeded

2972	threshold 2 exceeded

code memory fault path 31 charge air temperature sensor

code	description

3115	out of range, to high

3116	out of range, to low

code memory fault path 32 EGR temperature sensor

code	description

3215	out of range, to high

3216	out of range, to low

code memory fault path 33 EGR system

code	description

3332	EGR fault

code memory fault path 34 intake air - humidity sensor

code	description

3415	out of range, to high

3416	out of range, to low

code memory fault path 35 intake air - temperature sensor

code	description

3515	out of range, to high

3516	out of range, to low

code memory fault path 36 ambient air - temperature sensor

code	description

3604	no communication

3617	value not plausible

code memory fault path 37 SCR air pressure switch-over valve

code	description

3706	shortened to ground

3707	shortened to supply voltage

3709	interruption

code memory fault path 38 SCR catalyst - temperature sensors

code	description

3817	value not plausible

code memory fault path 39 AdBlue pump

code	description

3906	shortened to ground

3907	shortened to supply voltage

3909	interruption

code memory fault path 40 ecu internal fault

code	description

4024	auxiliary controller fault

4034	high side transistor open loop (PVB1)

4035	high side transistor open loop (PVB2)

4036	high side transistor open loop (PV5)

4037	cylinder amount not plausible

4038	main path starter driver

4040	starter driver detects not plausible signal

4047	fuel map not plausible

4048	cylinder amount and engine type not plausible

4049	parameter set for analog valves not valid

4050	wrong hardware detection

4051	checksum fault

4052	checksum fault 2 (driving parameters)

4053	checksum fault 3

4054	CAN - data file not valid

4056	overshoot time fault

4058	data set manipulated

4092	performance

code memory fault path 41 engine brakes

code	description

4149	parameters are set wrong

code memory fault path 42

code	description

4200	proportional valve 5, shorted to U_bat, + line, DKL-R

4201	proportional valve 5, shorted to ground, + line, DKL-R

4202	proportional valve 7, shorted to ground, - line, DKL

4203	proportional valve 7, interruption, DKL

4204	proportional valve 7, shorted to U_bat, - line, DKL

4205	throttle valve, postion sensor, out of range, to high

4206	throttle valve, postion sensor, out of range, to low

4207	DOC temperature sensor 1 out of range, to high

4208	DOC temperature sensor 1 out of range, to low

4209	DOC temperature sensor 2 out of range, to high

4210	DOC temperature sensor 2 out of range, to low

4211	pressure sensor before particulate filter (DPF) out of range, to high

4212	pressure sensor before particulate filter (DPF) out of range, to low

4213	pressure sensor behind particulate filter (DPF) out of range, to high

4214	pressure sensor behind particulate filter (DPF) out of range, to low

4215	particulate filter, differential pressure to high

4216	particulate filter, differential pressure to low

4217	throttle valve, offset to high

4218	throttle valve, reference drive not successful

4219	SCR dosing unit, air section is blocked / dirty

4220	SCR dosing unit, pessure duct is blocked / dirty

4221	diffusor heater, interruption

4222	diffusor heater, shorted to ground, - line

4223	diffusor heater, shorted to U_bat, - line

4224	internal fault, highside transistor PV5 highly resistive, DKL-R

4225	proportional valve  bank 1, shorted to U_bat, + line

4226	proportional valve  bank 1, shorted to ground, + line

4227	ecu internal fault, highside transistor highly resistive, PVB1

4228	SCRT system: component not present

4229	SCRT temp. sensors pair A (T-DOC-1, T-cat-2): implausible measurement

4230	SCRT temp. sensors pair B (T-DOC-2, T-cat-1): implausible measurement

4231	DPF: not present

4232	DPF press. sens. 1 implaus. measure

4233	DPF press. sens. 2 implaus. Measure

4234	NOx emissions, threshold 2 exceeded, cause unknown

4235	NOx emissions, urea quality

4236	NOx emissions, min. dose

4237	NOx sensor: implausible measure

4238	DPF: temp. too low

4239	MR sensor bank 1: Measured value too high / too low

4240	MR sensor bank 2: Measured value too high / too low

4241	SCR-RM sensor bank 1: Measured value too high / too low

4242	SCR-RM sensor bank 2: Measured value too high / too low

4243	SCR-RM sensor bank 3: Measured value too high / too low

4244	NOx sensor: Not ready error

4245	NOx emissions: NOx sensor faulty

4246	NOx emissions: Increased raw emissions

4247	NOx emissions: Faulty SCR-KAT

4248	NOx emissions: AdBlue quality / minimum dose

4249	NOx emissions: AdBlue quality / minimum dose faulty SCR-KAT

4250	Wasserpumpe: Masseschluss (-Leitung)

4251	Wasserpumpe: Leitungsbruch

4252	Wasserpumpe: UB-Schluss (-Leitung)

4253	SCR Speicherblase, Druck außerhalb zulässigem Bereich

4254	Auspuffklappe stufenlos: Allgemeiner Stellerfehler

4255	Auspuffklappe stufenlos: Masseschluss (-Leitung)

4256	Auspuffklappe stufenlos: UB-Schluss (-Leitung)

4257	Auspuffklappe stufenlos: Leitungsbruch

4258	DPF: Regeneration unzureichend

4259	Drosselklappe: Ladeluftdruck zu gering

4261	KDR Ventilstörung

4262	Steuergeräte interner Fehler: Bootblock defekt

4263	T4 Temp.-Sensor: Temperatur unplausibel

4264	Entleerung Druckkanal nicht durchgeführt

4265	Tankheizung am RM: Masseschluss (-Leitung)

4266	Tankheizung am RM: UB-Schluss (-Leitung)

4267	Tankheizung am RM: Leitungsbruch

4268	Tankheizung am RM: Komponente defekt

4269	Euromat3b-Tier4i: Tampering

4270	Harnstofftank-Füllstandssensor, Reservefüllstand unterschritten Tier4i

4271	Automatische VerdichtungsDetektion: Abbruch

4272	Automatische VerdichtungsDetektion: Timeout

4273	Steuergerätetemperatur, Messbereich überschritten

4274	Steuergerätetemperatur, Messbereich unterschritten

4275	Motor Durchdrehen aktiv

4276	Tier4i: unzulässiger Kaltbetrieb

code memory fault path 44 T4 temperature sensor

code	description

4415	out of range, to high

4416	out of range, to low

4422	temperature, to high

code memory fault path 48 control MV bank 1

code	description

4805	highside bank 1, shortened to supply voltage

4806	lowside bank 1, shortened to ground

code memory fault path 49 control MV bank 2

code	description

4905	highside bank 2, shortened to supply voltage

4906	lowside bank 2, shortened to ground

code memory fault path 50 injection valve 1

code	description

5026	no valve impact detected

5027	output driver fault

5028	shortened

code memory fault path 51 injection valve 2

code	description

5126	no valve impact detected

5127	output driver fault

5128	shortened

code memory fault path 52 injection valve 3

code	description

5226	no valve impact detected

5227	output driver fault

5228	shortened

code memory fault path 53 injection valve 4

code	description

5326	no valve impact detected

5327	output driver fault

5328	shortened

code memory fault path 54 injection valve 5

code	description

5426	no valve impact detected

5427	output driver fault

5428	shortened

code memory fault path 55 injection valve 6

code	description

5526	no valve impact detected

5527	output driver fault

5528	shortened

code memory fault path 56 injection valve 7

code	description

5626	no valve impact detected

5627	output driver fault

5628	shortened

code memory fault path 57 injection valve 8

code	description

5726	no valve impact detected

5727	output driver fault

5728	shortened

code memory fault path 58 SCR reductant pipe heating

code	description

5806	shortened to ground

5807	shortened to supply voltage

5809	interruption

5869	component defective, exhaust relevant

code memory fault path 59 analog valve 8 (tank heating valve)

code	description

5906	shortened to ground

5907	shortened to supply voltage

5909	interruption

5969	component defective, exhaust relevant

code memory fault path 60 AdBlue tank - liquid level sensor

code	description

6015	out of range, to high

6016	out of range, to low

6029	liquid level, to low

code memory fault path 61 AdBlue tank - temperature sensor

code	description

6115	out of range, to high

6116	out of range, to low

6169	component defective, exhaust relevant

code memory fault path 62 ecu internal fault - SCR modul

code	description

6293	high side transistor open loop

code memory fault path 63 OBD lamp (MI)

code	description

6327	output driver fault

code memory fault path 64 heater flange

code	description

6409	interruption

6432	EGR fault

code memory fault path 65 oil separator

code	description

6506	diagnosis line shortened, to ground

6564	oil separator fault

code memory fault path 66 EGR valve

code	description

6678	EGR actuator fault

6679	temperature diagnosis error

code memory fault path 67 AdBlue pressure system (UPS)

code	description

6721	pressure to low

6770	decrease of pressure at off-carriage to low

6777	control current out of tolerance (current of pump to high)

code memory fault path 68 SCR pressure system (UPS)

code	description

6820	pressure to high

6821	pressure to low

6868	air supply defective

code memory fault path 69 SCR system general

code	description

6949	parameterize error

6994	EGA switched off Mannheim function

6995	EGA switched off Wörth function

code memory fault path 70 analog valve 1

code	description

7006	shortened to ground

7007	shortened to supply voltage

7009	interruption

code memory fault path 71 analog valve 3

code	description

7106	shortened to ground

7107	shortened to supply voltage

7109	interruption

code memory fault path 72 analog valve 4

code	description

7206	shortened to ground

7207	shortened to supply voltage

7209	interruption

code memory fault path 73 analog valve 2

code	description

7306	shortened to ground

7307	shortened to supply voltage

7309	interruption

code memory fault path 74 analog valve 5

code	description

7405	shortened to supply voltage

7408	shortened to ground

code memory fault path 75 supply voltage

code	description

7542	over out of range

7543	under out of range

code memory fault path 76 analog valve 6

code	description

7606	shortened to ground

7607	shortened to supply voltage

7609	interruption

code memory fault path 77 PWM bank 1

code	description

7705	shortened to supply voltage

7708	shortened to ground

code memory fault path 78 PWM bank 2

code	description

7805	shortened to supply voltage

7808	shortened to ground

code memory fault path 79 PWM bank SCR modul

code	description

7905	shortened to supply voltage

7908	shortened to ground

code memory fault path 80 starter output

code	description

8005	starter relay exteraly shortened to battery voltage

8008	shortened to ground

8009	interruption

8033	starter relay permanently closed

8039	output stage with low impedance (main path or path of emergency)

8086	starter doesn´t lock up

code memory fault path 81 SCR air pressure stop valve

code	description

8106	shortened to ground

8107	shortened to supply voltage

8109	interruption

code memory fault path 82 analog valve 7 (AdBlue metering valve)

code	description

8206	shortened to ground

8207	shortened to supply voltage

8209	interruption

code memory fault path 83 AdBlue temperature sensor

code	description

8315	out of range, to high

8316	out of range, to low

code memory fault path 84 AdBlue pressure sensor

code	description

8415	out of range, to high

8416	out of range, to low

8417	value not plausible

code memory fault path 85 SCR air pressure - pressure sensor (MDU)

code	description

8515	out of range, to high

8516	out of range, to low

8517	value not plausible

code memory fault path 86 actual AdBlue consumption

code	description

8690	AdBlue consumption to high

8691	AdBlue consumption to low

code memory fault path 87 average AdBlue consumption

code	description

8790	AdBlue consumption to high

8791	AdBlue consumption to low

code memory fault path 88 SCR storage bubble

code	description

8820	pressure to high

8896	pressure out of range

code memory fault path 89 SCR air stop valve

code	description

8906	shorted to ground, - line

8907	shorted to U_bat, - line

8909	interruption

code memory fault path 90 charact. value cylinder 1

code	description

9044	idle balance limit cyl. 1

9045	selective adjustment limit cyl. 1

code memory fault path 91 charact. value cylinder 2

code	description

9144	idle balance limit cyl. 2

9145	selective adjustment limit cyl. 2

code memory fault path 92 charact. value cylinder 3

code	description

9244	idle balance limit cyl. 3

9245	selective adjustment limit cyl. 3

code memory fault path 93 charact. value cylinder 4

code	description

9344	idle balance limit cyl. 4

9345	selective adjustment limit cyl. 4

code memory fault path 94 charact. value cylinder 5

code	description

9444	idle balance limit cyl. 5

9445	selective adjustment limit cyl. 5

code memory fault path 95 charact. value cylinder 6

code	description

9544	idle balance limit cyl. 6

9545	selective adjustment limit cyl. 6

code memory fault path 96 charact. value cylinder 7

code	description

9644	idle balance limit cyl. 7

9645	selective adjustment limit cyl. 7

code memory fault path 97 charact. value cylinder 8

code	description

9744	idle balance limit cyl. 8

9745	selective adjustment limit cyl. 8

code memory fault path 98 selective cylinder adjustment

code	description

9846	time out

code memory fault path 99 immobiliser

code	description

9960	key amount limited to 8

9961	counter overflow

9962	X5 status reseted

9963	no transponder code via CAN

9964	no transpondercode via term. 50

9965	wrong key